AI-Powered Network Anomaly Detection

Built with Cisco Packet Tracer, Python & Scikit-learn

Figure 1 – Cisco Packet Tracer icon

# Part 1 — Build the Network in Cisco Packet Tracer

## Step 1 — Build the Basic Topology

Open Cisco Packet Tracer. From the bottom-left device menu, add:

- 2 × PCs

- 1 × Switch (2960)

- 1 × Router (1941)

Target topology:

PC1 ─── Switch ─── Router
PC2 ────┘

## Connect the Devices

Use Copper Straight-Through cables:

- PC1 FastEthernet0 → Switch Fa0/1

- PC2 FastEthernet0 → Switch Fa0/2

- Switch Fa0/24 → Router GigabitEthernet0/0

## Step 2 — Configure the Router

Click Router0 → CLI. Enter these commands one at a time:

enable
configure terminal
interface gigabitEthernet 0/0
ip address 192.168.1.1 255.255.255.0
no shutdown
exit
end

Note: After no shutdown, wait a few seconds — the link between the Switch and Router should turn green.

Figure 2 – Router interface brought up successfully

## Step 3 — Configure PC0

Click PC0 → Desktop → IP Configuration and enter:

IP Address:       192.168.1.10
Subnet Mask:      255.255.255.0
Default Gateway:  192.168.1.1

Leave DNS empty for now. Don't configure PC1 yet.

Figure 3 – PC0 IP configuration

## Step 4 — Configure PC1

Click PC1 → Desktop → IP Configuration and enter:

IP Address:       192.168.1.11
Subnet Mask:      255.255.255.0
Default Gateway:  192.168.1.1

Leave DNS as-is.

Figure 4 – PC1 IP configuration

## Step 5 — Test the Network

Before touching anything AI-related, confirm the basic network works. On PC0 → Desktop → Command Prompt, type:

ping 192.168.1.11

Reply from 192.168.1.11: bytes=32 time<1ms TTL=128
Reply from 192.168.1.11: bytes=32 time<1ms TTL=128
Reply from 192.168.1.11: bytes=32 time<1ms TTL=128

A Reply response confirms the network is working. ✅

Figure 5 – Successful ping between PC0 and PC1

# Part 2 — Generate Network Traffic Data

## Step 6 — Generate Baseline Traffic

For the AI to detect an anomaly, we first need network data. On PC0 → Command Prompt, run:

ping 192.168.1.11 -n 20

This sends 20 packets to PC1. Wait until it finishes.

Figure 6 – 20-packet baseline ping

## Step 7 — Read Router Traffic Statistics

Close the PC window, then go to Router0 → CLI:

enable
show interfaces gigabitEthernet 0/0

Look for the lines containing: packets input, bytes, packets output, errors.

Figure 7 – Router interface counters

## Step 8 — Create Abnormal Traffic

Now push the network to behave differently, so the AI model has something to detect. On PC0 → Desktop → Command Prompt, run:

ping 192.168.1.11 -n 100

Let it finish completely.

Figure 8 – 100-packet stress ping (part 1)

Figure 9 – 100-packet stress ping (part 2)

## Step 9 — Check the Router Counters Again

Back on Router0 → CLI, run:

show interfaces gigabitEthernet 0/0

Look at: packets input, bytes, packets output, errors — especially 0 input errors, 0 output errors, 0 drops.

Figure 10 – Router counters after the stress ping

The router still shows 5-minute input/output rates of 0 bits/sec, 0 packets/sec. That's because PC0 → PC1 traffic stays inside the switch — it never passes through Router0. The 100-ping test therefore gave the router nothing to measure. We'll fix that next.

## Step 10 — Route Traffic Through the Router

On PC0 → Desktop → Command Prompt, run:

ping 192.168.1.1 -n 100

PC0 (192.168.1.10)
     ↓
   Switch
     ↓
Router (192.168.1.1)

This traffic reaches Router0, so its interface counters can finally record it.

Figure 11 – Ping results routed through Router0

## Step 11 — Collect the First Data Point

Open Notepad and create a file named network_data.csv with the header row and first sample:

sample,packets_input,bytes_input,packets_output,bytes_output,input_errors,output_errors,drops
1,100,12800,100,12800,0,0,0

This is the first normal network observation, taken from the router's interface counters:

Figure 12 – Router counters used for sample 1

## Step 12 — Collect the Second Data Point

Collect another normal traffic observation. On PC0 → Desktop → Command Prompt, run:

ping 192.168.1.1 -n 50

Then on Router0 → CLI, run:

show interfaces gigabitEthernet 0/0

and compare the updated packets input / bytes / packets output / errors.

Note: Router counters are cumulative — don't record the running total as the new sample. Use the difference between readings. For example, if the previous total was 100 packets input and the new total is 150, this sample generated 150 − 100 = 50 packets.

Figure 13 – Router counters for sample 2

For this project, a simulated dataset built from the same metrics observed in Packet Tracer is used to provide enough variety to train the model — this is documented as simulated / sample telemetry rather than a live capture.

# Part 3 — Build the Dataset

## Step 13 — Complete Dataset

The full 30-sample dataset saved to network_data.csv:

| Sample | Packets In | Bytes In | Packets Out | Bytes Out | Input Errors | Output Errors | Drops |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 12800 | 100 | 12800 | 0 | 0 | 0 |
| 2 | 50 | 6400 | 50 | 6400 | 0 | 0 | 0 |
| 3 | 80 | 10240 | 80 | 10240 | 0 | 0 | 0 |
| 4 | 65 | 8320 | 65 | 8320 | 0 | 0 | 0 |
| 5 | 120 | 15360 | 120 | 15360 | 0 | 0 | 0 |
| 6 | 90 | 11520 | 90 | 11520 | 0 | 0 | 0 |
| 7 | 75 | 9600 | 75 | 9600 | 0 | 0 | 0 |
| 8 | 110 | 14080 | 110 | 14080 | 0 | 0 | 0 |
| 9 | 95 | 12160 | 95 | 12160 | 0 | 0 | 0 |
| 10 | 130 | 16640 | 130 | 16640 | 0 | 0 | 0 |
| 11 | 70 | 8960 | 70 | 8960 | 0 | 0 | 0 |
| 12 | 85 | 10880 | 85 | 10880 | 0 | 0 | 0 |
| 13 | 105 | 13440 | 105 | 13440 | 0 | 0 | 0 |
| 14 | 115 | 14720 | 115 | 14720 | 0 | 0 | 0 |
| 15 | 125 | 16000 | 125 | 16000 | 0 | 0 | 0 |
| 16 | 100 | 12800 | 100 | 12800 | 0 | 0 | 0 |
| 17 | 135 | 17280 | 135 | 17280 | 0 | 0 | 0 |
| 18 | 145 | 18560 | 145 | 18560 | 0 | 0 | 0 |
| 19 | 155 | 19840 | 155 | 19840 | 0 | 0 | 0 |
| 20 | 165 | 21120 | 165 | 21120 | 0 | 0 | 0 |
| 21 | 450 | 57600 | 440 | 56320 | 8 | 5 | 3 |
| 22 | 520 | 66560 | 510 | 65280 | 12 | 8 | 6 |
| 23 | 600 | 76800 | 590 | 75520 | 15 | 10 | 8 |
| 24 | 700 | 89600 | 680 | 87040 | 20 | 15 | 12 |
| 25 | 800 | 102400 | 780 | 99840 | 25 | 18 | 15 |
| 26 | 75 | 9600 | 75 | 9600 | 0 | 0 | 0 |
| 27 | 90 | 11520 | 90 | 11520 | 0 | 0 | 0 |
| 28 | 680 | 87040 | 660 | 84480 | 18 | 12 | 10 |
| 29 | 750 | 96000 | 730 | 93440 | 22 | 16 | 14 |
| 30 | 850 | 108800 | 820 | 104960 | 30 | 20 | 18 |

Rows shaded red contain non-zero errors or drops and represent abnormal traffic.

What the dataset represents:

- Samples 1–20: normal traffic

- Samples 21–25: abnormal traffic

- Samples 26–27: normal traffic again

- Samples 28–30: abnormal traffic

Normal:                         Anomaly:
100 packets                    800+ packets
0 errors            ──▶        20+ errors
0 drops                        10+ drops

This variation is what lets the model demonstrate real anomaly detection, instead of learning from only two or three points.

Project data flow so far:

Cisco Packet Tracer
      ↓
Simulated Network Metrics
      ↓
   network_data.csv
      ↓
     Python
      ↓
  Machine Learning
      ↓
  Anomaly Detection
      ↓
  Normal / Anomaly

# Part 4 — AI Analysis in Python

## Step 14 — Install the Python AI Libraries

Open Command Prompt / PowerShell and run:

pip install pandas scikit-learn

- Pandas → reads network_data.csv

- Scikit-learn → runs the AI anomaly-detection algorithm

Then confirm the Python version:

python --version

Figure 14 – Library installation confirmed

## Step 15 — AI Anomaly Detection

Script: ai_network_analytics.py

Uses Isolation Forest to flag unusual network behavior from packet count, bytes, input/output errors, and packet drops, classifying each sample as NORMAL or ANOMALY.

## Step 16 — Run Anomaly Detection

python ai_network_analytics.py

The script analyzes network_data.csv and writes the results to ai_results.csv.

## Step 17 — Predictive Analytics

Script: predictive_analytics.py — uses Linear Regression to forecast future packet traffic:

Sample 31 → 590 packets
Sample 32 → 612 packets
Sample 33 → 633 packets
Sample 34 → 655 packets
Sample 35 → 677 packets

Figure 15 – Predictive analytics output

## Step 18 — Combined AI Dashboard

Script: ai_network_dashboard.py — combines Anomaly Detection + Predictive Analytics into a single program.

## Step 19 — Improved Anomaly Detection

Added a hybrid approach:

Isolation Forest
       +
Network Behavior Thresholds
       ↓
  Final Anomaly Decision

Each anomaly now includes a plain-language reason, such as:

- High packet volume

- High input errors

- Packet drops

- ML detected unusual pattern

## Step 20 — Run the Final AI Dashboard

python ai_network_dashboard.py

Figure 16 – Dashboard: anomaly detection table

Figure 17 – Dashboard: detailed anomaly reasons

Figure 18 – Dashboard: summary, prediction & AI analysis

Figure 19 – Dashboard: final analysis output

Final result: 21 normal samples, 9 anomaly samples. The system also predicted increasing future traffic and raised an AI alert for anomalous activity.

## Step 21 — Visualization

Script: network_graph.py — generates anomaly_detection.png and traffic_prediction.png.

Figure 20 – Anomaly detection scatter plot (blue = normal, orange = anomaly)

# Final Project Summary

Cisco Packet Tracer
      ↓
  Network Simulation
      ↓
   Network Dataset
      ↓
      Python
      ↓
┌───────────────────────┐
│  AI Network Analytics  │
├───────────────────────┤
│  Anomaly Detection     │
│  Predictive Analytics  │
│  Visualization         │
└───────────────────────┘

Project status: COMPLETED ✅